Start Up Procedure for Propane Grinders:

- Turn on Gas

- Increase throttle slightly

- Turn key

- Once motor turns over, increase throttle until the motor is running around 2,500 rpms

- Push the “engagement” switch

- Immediately begin swinging left to right.

When ready to stop

- Decrease throttle to 1,200 rpms

- Turn off gas

- Continue swinging machine side to side until the motor dies and the operator is unable to move it anymore.

- Turn the key to the off position