### Operating Instructions 1-10DE99

Lubrication

Section 9

9.1	Lubrication

### Operating Instructions 1-10DE99

Lubrication

9.1 Lubrication

The bearing unit that is located directly behind the blast wheel outside the blast housing must receive periodic lubrication.  This unit operates at a relatively high temperature (200 degrees Fahrenheit) and requires the use of a high temperature, lithium based grease. Mobilux 77 EP-2 is one example.

The bearing unit should receive several pumps of this high temperature, lithium based grease at least every 50 hours.  Several pumps of grease from a standard grease gun should be sufficient.

The grease needs to be inserted into the grease fittings on the bearing unit.  Once the unit is greased, run the blast wheel for 10 - 15 minutes to purge any excess grease.

Do not over-grease this unit.