Zelio Logic 2 – Programmer

Changing the Pulse timing interval

First, before changing the programmer…

Turn on the Compressor and time how long it takes for it to get from empty to 70-80 PSI.

Steps:

Push Menu/OK button once.

Push Arrow Down button to “PARAMETER”

Push Menu/OK button once to select.

Push Arrow Up button once to get to display that shows pulse interval in lower left corner.

Push Arrow Right button TWICE  until B= (value) s is blinking.   The (value) represents what the pulse interval in seconds is currently set at.

Change the (value) using the Up & Down Arrows to the time it takes the compressor to get from 0 PSI or 0 BAR to 75 – 80 psi.   To convert BAR to PSI = multiply the BAR reading by 14, and that equals PSI.

Push Menu/OK button once.  Display will show: 		Confirm Changes?

Yes

No

Use Arrow buttons to get to YES and push Menu/OK.

Then push Menu/OK until display returns to the original screen display.

I am pretty sure these are as exact as it took to make the changes.

Hope this helps, David Stroud